# Command Reference

Automotive Body Control Module (BCM) Simulator

| Document ID | BCM-CMD-001 |
| --- | --- |
| Version | 1.0 |
| Software version | 1.0.0 |
| Repository | C:\Users\Akshansh\bcm-simulator |

Two rules that apply to every command 1. Run them from the repository root, not from a subfolder: cd C:\Users\Akshansh\bcm-simulator 2. Use the long form for bcm.ps1 so PowerShell's execution policy does not block it: powershell -ExecutionPolicy Bypass -File tools\bcm.ps1 <action> Throughout this document that is shortened to 'tools\bcm.ps1 <action>' for readability.

## Quick reference

| Command | What it does | Board? |
| --- | --- | --- |
| tools\bcm.ps1 build | Compile the firmware | no |
| tools\bcm.ps1 test | 189 host unit tests | no |
| tools\bcm.ps1 sil | 15 vehicle scenarios | no |
| tools\bcm.ps1 run | Run the firmware in QEMU | no |
| tools\bcm.ps1 debug | QEMU halted, waiting for GDB | no |
| tools\bcm.ps1 docs | Generate the API reference | no |
| tools\bcm.ps1 clean | Delete firmware build output | no |
| tools\bcm.ps1 ports | List serial ports | no |
| tools\bcm.ps1 flash | Programme the board | YES |
| tools\bcm.ps1 verify | Prove the CPU is executing | YES |
| tools\bcm.ps1 talk | Exercise the protocol over serial | YES |
| tools\bcm.ps1 desktop | Build and open the Windows GUI | GUI |

# 1. Build and clean

tools\bcm.ps1 build

| Purpose | Compiles the firmware for the STM32F103 and prints the image size. Fetches the vendor HAL and FreeRTOS automatically the first time. |
| --- | --- |
| Hardware | Not required. |

Typical output:

arm-none-eabi-g++ ... -o build/bcm_firmware.elf text data bss dec hex filename 24564 124 8456 33144 8178 build/bcm_firmware.elf

text = flash used (38 % of 64 kB); bss = RAM used (43 % of 20 kB).

Output lands in firmware/build/ as .elf, .hex and .bin.

tools\bcm.ps1 clean

| Purpose | Deletes the firmware build directory. Use it if a build behaves oddly after switching branches. |
| --- | --- |
| Hardware | Not required. |

Typical output:

rm -rf build

## Equivalent make targets

bcm.ps1 wraps these. Use them directly only if you have already put the toolchain on PATH yourself.

| Directory | Target | Effect |
| --- | --- | --- |
| firmware/ | make | Build .elf, .hex, .bin and print the size |
| firmware/ | make deps | Clone STM32CubeF1 and FreeRTOS (one-off) |
| firmware/ | make size | Print the image size only |
| firmware/ | make flash | st-flash write (NOT used - bcm.ps1 uses OpenOCD) |
| firmware/ | make monitor | Print serial terminal hints |
| firmware/ | make clean | Remove firmware/build/ |
| tests/ | make | Build the unit test binary |
| tests/ | make run | Build and run the unit tests |
| tests/ | make deps | Fetch the Catch2 single header |
| tests/ | make clean | Remove tests/build/ |
| sil/ | make | Build the SIL harness |
| sil/ | make run | Build and run all scenarios |
| sil/ | make list | List scenario names |
| sil/ | make clean | Remove sil/build/ |

# 2. Testing (no board needed)

tools\bcm.ps1 test

| Purpose | Builds and runs the host unit test suite - every module tested in isolation against hostile inputs. |
| --- | --- |
| Hardware | Not required. Runs natively on the PC. |

Typical output:

All tests passed (3630 assertions in 189 test cases) PASS: all unit tests green.

Exits non-zero on failure, so it can gate a CI pipeline.

Tests the same source files the firmware links, not a copy.

tools\bcm.ps1 sil

| Purpose | Runs the Software-in-the-Loop scenarios: the whole BCM driven through vehicle situations over simulated time. |
| --- | --- |
| Hardware | Not required. |

Typical output:

PASS Brake lamp overrides everything SRS-BRK-001, SAFETY-003 PASS Doors never lock while one is open SRS-DOOR-002, SAFETY-004 scenarios: 15 passed: 15 failed: 0 checks: 79 passed: 79 failed: 0 RESULT: PASS

Variants:

tools\bcm.ps1 sil -SilArgs --verbose # list passing checks too tools\bcm.ps1 sil -SilArgs --list # print scenario names tools\bcm.ps1 sil -SilArgs brake-priority # run one scenario tools\bcm.ps1 sil -SilArgs door-open-guard

tools\bcm.ps1 desktop -SilArgs test

| Purpose | Runs the C# conformance suite: proves the Windows tool and the firmware agree on the protocol, byte for byte. |
| --- | --- |
| Hardware | Not required. |

Typical output:

Passed! - Failed: 0, Passed: 17, Skipped: 0, Total: 17

# 3. Running without hardware (QEMU)

tools\bcm.ps1 run

| Purpose | Runs the real firmware image in an STM32F103 emulator. The heartbeat LED is printed as text. |
| --- | --- |
| Hardware | Not required. |

Typical output:

Board: 'BluePill' (BluePill STM32F103C8T6). [led:red off] [led:red on] [led:red off]

Press Ctrl+C to stop.

Add -Seconds 8 to stop automatically after 8 seconds.

QEMU is NOT cycle-accurate: a 500 ms delay measures about 722 ms. Use it for logic, never for timing.

tools\bcm.ps1 debug

| Purpose | Starts QEMU with the processor halted, waiting for a debugger on port 1234. Press F5 in VS Code to attach. |
| --- | --- |
| Hardware | Not required. |

Typical output:

==> QEMU halted at reset, waiting for GDB on localhost:1234

Open the 'firmware' folder in VS Code, not the repository root.

-Attach drops straight into a console GDB session instead.

-GdbPort 4321 uses a different port.

# 4. Hardware

tools\bcm.ps1 ports

| Purpose | Lists the serial ports Windows can see. Use it to find your USB-UART adapter before connecting. |
| --- | --- |
| Hardware | The UART adapter must be plugged in. |

Typical output:

Serial ports: COM13

The ST-Link does NOT appear here - it is not a serial port.

tools\bcm.ps1 flash

| Purpose | Builds, then programmes the board over SWD using the ST-Link, and verifies what was written. |
| --- | --- |
| Hardware | ST-Link connected; the board's power LED must be lit. |

Typical output:

Info : SWD DPIDR 0x2ba01477 Info : [stm32f1x.cpu] Cortex-M3 r2p0 processor detected ** Programming Finished ** ** Verified OK ** ==> Flash OK. PC13 should now blink at 1 Hz.

-Method swd forces SWD; -Method uart uses the ROM bootloader instead (needs BOOT0 = 1 and the door-lock LED unplugged from PA9).

'unable to connect to the target' almost always means the board is unpowered or SWDIO/SWCLK are swapped.

tools\bcm.ps1 verify

| Purpose | Reads the processor's output register live over SWD and confirms the heartbeat pin is toggling. This proves the CPU is EXECUTING, not merely that bytes reached flash. |
| --- | --- |
| Hardware | ST-Link connected. |

Typical output:

==> Sampling GPIOC->ODR every 250 ms for 3 s ... PC13: *o**oo**o**o ('*' = LED on, 'o' = LED off) PASS: PC13 is toggling (6 high / 6 low samples) - firmware is running.

'stuck HIGH/LOW' means the chip is powered but the program is not running - usually a crash or a failed clock configuration.

tools\bcm.ps1 talk

| Purpose | Speaks the diagnostic protocol to the board over the serial link and checks six commands, including two deliberately malformed ones. |
| --- | --- |
| Hardware | USB-UART adapter on PB6/PB7. Close the GUI first - only one program may hold the port. |

Typical output:

[ OK ] PING echo verified [ OK ] GET_VERSION v1.0.0 [ OK ] GET_STATUS lamps=0x0000 switches=0x00 [ OK ] GET_BATTERY 27.6% of full scale [ OK ] bad command answered UNKNOWN_CMD [ OK ] short SETLAMP answered BAD_LENGTH PASS: 6/6 protocol checks OK.

-Port COM13 selects a port explicitly if several are present.

-Baud 115200 is the default and rarely needs changing.

# 5. The Windows diagnostic tool

tools\bcm.ps1 desktop

| Purpose | Builds and launches the graphical diagnostic tool. Select the COM port and press Connect. |
| --- | --- |
| Hardware | Board and UART adapter for live data; the window opens either way. |

Typical output:

==> Building the diagnostic tool Build succeeded. ==> Launching. Pick the COM port and press Connect.

Do not double-click the .exe desktop\BcmDiagnosticTool\bin\Release\net8.0-windows\BcmDiagnosticTool.exe fails with 'You must install .NET to run this application'. The SDK here is installed per-user, so the program cannot locate its runtime by itself. Launch it through bcm.ps1, which sets DOTNET_ROOT - or set that variable yourself: $env:DOTNET_ROOT = "$env:USERPROFILE\.dotnet" & .\desktop\BcmDiagnosticTool\bin\Release\net8.0-windows\BcmDiagnosticTool.exe --connect COM13

Direct .NET commands:

$env:DOTNET_ROOT = "$env:USERPROFILE\.dotnet" $env:PATH = "$env:DOTNET_ROOT;$env:PATH" dotnet build desktop\BcmDiagnosticTool.sln -c Release dotnet test desktop\BcmDiagnosticTool.sln -c Release dotnet clean desktop\BcmDiagnosticTool.sln

# 6. Documentation

tools\bcm.ps1 docs

| Purpose | Generates the API reference from the source comments into docs/api/html/index.html. |
| --- | --- |
| Hardware | Not required. |

Typical output:

==> Generating the API reference no warnings ==> Done: ...\docs\api\html\index.html

-SilArgs open also opens it in your browser.

docs/api/ is gitignored - it is generated output.

Regenerating the Word documents:

python tools\make_test_procedure.py # -> docs/test/BCM-TestProcedure.docx python tools\make_command_reference.py # -> docs/CommandReference.docx

## Where the written documents live

| Document | ID | Path |
| --- | --- | --- |
| Project Report | BCM-PRJ-001 | docs/ProjectReport.md |
| Requirements | BCM-SRS-001 | docs/requirements/SRS.md |
| Architecture | BCM-ARC-001 | docs/architecture/ArchitectureDocument.md |
| Protocol (ICD) | BCM-ICD-001 | docs/protocol/ICD.md |
| UML model | BCM-UML-001 | docs/uml/UML.md |
| Test report | BCM-TST-001 | docs/test/TestReport.md |
| Test procedure (Word) | BCM-ATP-001 | docs/test/BCM-TestProcedure.docx |
| User manual | BCM-MAN-001 | docs/manual/UserManual.md |
| Command reference (Word) | BCM-CMD-001 | docs/CommandReference.docx |
| Release notes | - | docs/ReleaseNotes.md |

# 7. All bcm.ps1 parameters

The action is positional and comes first. Everything else is optional and only applies to certain actions.

| Parameter | Values | Default | Applies to |
| --- | --- | --- | --- |
| (action) | build, run, debug, clean, flash, ports, verify, test, talk, sil, desktop, docs | run | - |
| -Seconds | whole number | 0 (forever) | run |
| -GdbPort | TCP port | 1234 | debug |
| -Attach | switch | off | debug |
| -Port | COM13 etc. | auto-detect | flash, talk |
| -Baud | whole number | 115200 | flash (uart), talk |
| -Method | auto, swd, uart | auto | flash |
| -SilArgs | list of strings | empty | sil, desktop, docs |

## Examples

tools\bcm.ps1 run -Seconds 8 # emulate for 8 s then stop tools\bcm.ps1 debug -Attach # QEMU + console GDB tools\bcm.ps1 flash -Method uart -Port COM13 tools\bcm.ps1 talk -Port COM13 tools\bcm.ps1 sil -SilArgs --verbose tools\bcm.ps1 docs -SilArgs open tools\bcm.ps1 desktop -SilArgs test

# 8. One-time setup commands

Everything installs per-user. No administrator rights are needed and nothing is added to PATH - bcm.ps1 locates each tool itself.

# Embedded toolchain, build tools, debugger xpm install --global @xpack-dev-tools/arm-none-eabi-gcc@latest xpm install --global @xpack-dev-tools/windows-build-tools@latest xpm install --global @xpack-dev-tools/openocd@latest # Host compiler, for the unit tests and the SIL xpm install --global @xpack-dev-tools/mingw-w64-gcc@latest # Emulator - MUST be the 2.8 gnuarmeclipse build, not 9.x xpm install --global @xpack-dev-tools/qemu-arm@2.8.0-13.1 # .NET, for the Windows diagnostic tool $s = "$env:TEMP\dotnet-install.ps1" Invoke-WebRequest https://dot.net/v1/dotnet-install.ps1 -OutFile $s -UseBasicParsing & $s -Channel 8.0 -InstallDir "$env:USERPROFILE\.dotnet" -NoPath # Doxygen - portable, just unzip it Invoke-WebRequest https://www.doxygen.nl/files/doxygen-1.12.0.windows.x64.bin.zip ` -OutFile "$env:TEMP\doxygen.zip" -UseBasicParsing Expand-Archive "$env:TEMP\doxygen.zip" "$env:USERPROFILE\.local\doxygen" # Python helper used to generate the Word documents python -m pip install --user python-docx # Fetch the vendor HAL and FreeRTOS (not stored in the repository) cd firmware; make deps

QEMU version matters Upstream QEMU 9.x maps the STM32F1 clock peripheral as a stub that always reads zero, so the firmware hangs forever waiting for the oscillator and gives no clue why. Only the gnuarmeclipse 2.8 build emulates this chip properly.

# 9. Advanced: OpenOCD and GDB directly

bcm.ps1 wraps these. Use them when you want on-chip debugging or to inspect a register by hand.

Start an OpenOCD session (leaves a GDB server on port 3333):

$R = "$env:APPDATA\xPacks\@xpack-dev-tools\openocd\0.12.0-7.1\.content" & "$R\bin\openocd.exe" -s "$R\openocd\scripts" ` -f interface/stlink.cfg -c "transport select swd" ` -f target/stm32f1x.cfg

Attach GDB to a running target:

$G = "$env:APPDATA\xPacks\@xpack-dev-tools\arm-none-eabi-gcc\15.2.1-1.1.1\.content\bin" cd firmware & "$G\arm-none-eabi-gdb.exe" build\bcm_firmware.elf (gdb) target extended-remote localhost:3333 (gdb) interrupt # stop the processor (gdb) backtrace # where is it? (gdb) break bsp.cpp:37 (gdb) continue (gdb) info registers pc

Read a peripheral register without stopping the program:

# inside an OpenOCD session, or via -c on the command line mdw 0x4001100c # GPIOC output register (bit 13 = heartbeat LED) mdw 0x40010c0c # GPIOB output register (8 of the lamps) mdw 0x40010808 # GPIOA input register (bits 0-5 = the buttons)

These are how the sleep-state diagnosis in the test procedure was made: reading the lamp outputs directly showed only the door-lock lamp lit, which is the signature of a sleeping BCM.

## Git commands used in this project

git status --short # what has changed git log --oneline # commit history git diff # unstaged changes git add -A # stage everything git commit # record a commit git checkout -b <branch> # start a branch git merge --ff-only <branch> # fast-forward merge

# 10. Recipes

## Full regression, in order

tools\bcm.ps1 test # 1. unit tests tools\bcm.ps1 sil # 2. scenarios tools\bcm.ps1 desktop -SilArgs test # 3. protocol conformance tools\bcm.ps1 build # 4. firmware compiles tools\bcm.ps1 flash # 5. programme the board tools\bcm.ps1 verify # 6. CPU is running tools\bcm.ps1 talk # 7. protocol on hardware tools\bcm.ps1 docs # 8. documentation builds

## I changed the firmware - what do I run?

tools\bcm.ps1 test # did I break a module? tools\bcm.ps1 sil # did I break vehicle behaviour? tools\bcm.ps1 flash # put it on the board tools\bcm.ps1 verify # is it actually running?

## I have no board today

tools\bcm.ps1 test tools\bcm.ps1 sil tools\bcm.ps1 run tools\bcm.ps1 desktop -SilArgs test

## Something is wrong with the board

tools\bcm.ps1 ports # is the UART adapter there? tools\bcm.ps1 verify # is the CPU executing? tools\bcm.ps1 talk # does it answer the protocol? tools\bcm.ps1 flash # reprogramme it

If the lamps do not respond to the buttons That is usually not a fault. The BCM starts in SLEEP, where every lamp except the door-lock indicator is deliberately kept off. Tap the ignition button BRIEFLY - under 0.8 seconds - to wake it. Holding the button longer changes the lighting mode instead and does not wake the vehicle. See BCM-ATP-001, test case TC-C-01.

Related documents: BCM-ATP-001 (test procedure), BCM-PRJ-001 (project report), BCM-MAN-001 (user manual), BCM-ICD-001 (protocol).