# Acceptance Test Procedure

Automotive Body Control Module (BCM) Simulator

| Document ID | BCM-ATP-001 |
| --- | --- |
| Version | 1.0 |
| Software version | 1.0.0 |
| Target | STM32F103C8T6 'Blue Pill', 72 MHz Cortex-M3 |
| Tester | ________________________________ |
| Date | ________________________________ |

## How to use this document

Work through the parts in order. Each test case has numbered steps, the exact result to expect, and a Pass/Fail box to tick. If a step fails, stop and check Part F (Troubleshooting) before continuing - later tests usually depend on earlier ones.

Part A runs entirely on the PC. No board required.

Part B connects and programmes the hardware.

Part C is the hands-on functional test.

Part D covers the safety-critical behaviour.

Part E uses the Windows diagnostic tool.

Part F is troubleshooting; Part G is the sign-off sheet.

READ THIS FIRST — the most common mistake The BCM starts in SLEEP, and a sleeping vehicle deliberately keeps every lamp off except the door-lock indicator. If you press the indicator or hazard buttons before waking it, NOTHING will light and the hardware will look broken when it is working correctly. Always run TC-C-01 (wake the BCM) first. The ignition button is dual-function: a SHORT tap (under 0.8 s) toggles power on and off, while HOLDING it for 0.8 s or more changes the lighting mode instead and does not toggle power. If you hold it too long, the BCM never wakes.

# Prerequisites

## Equipment

| Item | Notes | Present? |
| --- | --- | --- |
| STM32F103C8 'Blue Pill' | Power LED must light when powered | ☐ |
| ST-Link V2 programmer | For flashing and SWD debug | ☐ |
| USB-UART adapter | 3.3 V logic. NOT the same as the ST-Link | ☐ |
| 10 LEDs + 220 Ω–1 kΩ resistors | 10 kΩ makes an LED invisible | ☐ |
| 6 push buttons + 10 kΩ resistors | Wired +3.3 V → button → GPIO → 10 kΩ → GND | ☐ |
| Potentiometer | Simulates battery voltage, on PA6 | ☐ |
| Buzzer | On PB8 | ☐ |
| Windows PC | With this repository checked out | ☐ |

## Wiring

ST-Link Blue Pill UART adapter Blue Pill ------- --------- ------------ --------- 3.3V ----> 3V3 RX <----- PB6 (BCM TX) GND ----> GND TX -----> PB7 (BCM RX) SWDIO ----> DIO (PA13) GND <----> GND SWCLK ----> CLK (PA14)

RX and TX cross over. Do not connect the UART adapter's power pin if the board is already powered.

## I/O map

| Pin | Function | Pin | Function |
| --- | --- | --- | --- |
| PA0 | Ignition button | PB0 | Ignition lamp |
| PA1 | Left indicator button | PB1 | DRL lamp |
| PA2 | Right indicator button | PB8 | Buzzer |
| PA3 | Hazard button | PB10 | Low beam |
| PA4 | Brake button | PB11 | High beam |
| PA5 | Door lock button | PB12 | Left indicator lamp |
| PA6 | Potentiometer (ADC) | PB13 | Right indicator lamp |
| PA8 | Reverse lamp | PB14 | Hazard lamp |
| PA9 | Door lock lamp | PB15 | Brake lamp |
| PA13/PA14 | SWD — keep free | PB6/PB7 | Diagnostic UART |

# Part A — Software tests (no hardware needed)

Run these from the repository root in PowerShell. Every command returns a non-zero exit code on failure, so they can also be used in continuous integration.

### TC-A-01 — Unit tests

| Requirement | SRS-MNT-002 and all module requirements |
| --- | --- |
| Precondition | Repository checked out. No hardware required. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Open PowerShell in the repository root. | PowerShell prompt is at C:\...\bcm-simulator | ☐ P ☐ F |
| 2 | Run: tools\bcm.ps1 test | The suite builds, then runs. | ☐ P ☐ F |
| 3 | Read the final two lines. | 'All tests passed (3630 assertions in 189 test cases)' followed by 'PASS: all unit tests green.' | ☐ P ☐ F |

### TC-A-02 — Software-in-the-Loop scenarios

| Requirement | SRS-LIGHT-*, SRS-IND-*, SRS-DOOR-*, SRS-PWR-*, SRS-SAFETY-* |
| --- | --- |
| Precondition | TC-A-01 passed. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Run: tools\bcm.ps1 sil | 15 scenarios listed, each marked PASS. | ☐ P ☐ F |
| 2 | Check every scenario line reads PASS. | 'scenarios: 15 passed: 15 failed: 0' and 'RESULT: PASS' | ☐ P ☐ F |
| 3 | Optional: tools\bcm.ps1 sil -SilArgs --verbose | Each individual check is listed as well. | ☐ P ☐ F |
| 4 | Optional: tools\bcm.ps1 sil -SilArgs door-open-guard | Runs that one scenario only: 1 passed, 5 checks. | ☐ P ☐ F |

### TC-A-03 — Protocol conformance (C# vs firmware)

| Requirement | BCM-ICD-001 |
| --- | --- |
| Precondition | None. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Run: tools\bcm.ps1 desktop -SilArgs test | The solution builds with 0 errors. | ☐ P ☐ F |
| 2 | Read the summary line. | 'Passed! - Failed: 0, Passed: 17, Skipped: 0, Total: 17' | ☐ P ☐ F |

### TC-A-04 — Emulated run (QEMU)

| Requirement | SRS-PERF-003 |
| --- | --- |
| Precondition | None. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Run: tools\bcm.ps1 run | QEMU reports board 'BluePill' / device STM32F103C8. | ☐ P ☐ F |
| 2 | Observe the console for a few seconds. | Alternating '[led:red on]' and '[led:red off]' lines. | ☐ P ☐ F |
| 3 | Press Ctrl+C to stop. | Returns to the prompt. | ☐ P ☐ F |

Note on QEMU timing QEMU is not cycle-accurate: a 500 ms delay measures around 722 ms there. Use it to check logic, never to measure timing. The real hardware is exact.

# Part B — Hardware setup and programming

### TC-B-01 — Power and connections

| Requirement | — |
| --- | --- |
| Precondition | Board wired as shown in Prerequisites. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Connect the ST-Link to the PC. | Windows detects the ST-Link (no driver warning). | ☐ P ☐ F |
| 2 | Look at the board's red power LED. | The power LED is LIT. If dark, stop - see Part F. | ☐ P ☐ F |
| 3 | Run: tools\bcm.ps1 ports | The UART adapter's COM port is listed, e.g. COM13. | ☐ P ☐ F |

### TC-B-02 — Programme the firmware

| Requirement | SRS-PERF-003 |
| --- | --- |
| Precondition | TC-B-01 passed. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Run: tools\bcm.ps1 flash | Builds, then connects over SWD. | ☐ P ☐ F |
| 2 | Read the size line. | text ≈ 24564, data ≈ 124, bss ≈ 8456 (38 % flash, 43 % RAM). | ☐ P ☐ F |
| 3 | Read the final lines. | '** Verified OK **' and '==> Flash OK.' | ☐ P ☐ F |

### TC-B-03 — Confirm the CPU is executing

| Requirement | SRS-SAFETY-006 |
| --- | --- |
| Precondition | TC-B-02 passed. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Run: tools\bcm.ps1 verify | 'PASS: PC13 is toggling - firmware is running.' This reads the output register live over SWD, so it proves the processor is executing, not merely that bytes reached flash. | ☐ P ☐ F |
| 2 | Look at the small LED on the Blue Pill itself. | The on-board LED blinks about once per second. | ☐ P ☐ F |

### TC-B-04 — Diagnostic protocol over the real link

| Requirement | SRS-COM-001, SRS-COM-002, SRS-COM-003 |
| --- | --- |
| Precondition | TC-B-03 passed. UART adapter connected to PB6/PB7. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Close the diagnostic tool if it is open (it holds the port). | Only one program can use the COM port at a time. | ☐ P ☐ F |
| 2 | Run: tools\bcm.ps1 talk | Six checks run. | ☐ P ☐ F |
| 3 | Check all six lines. | All six read [ OK ], ending 'PASS: 6/6 protocol checks OK.' GET_BATTERY shows a percentage that follows the potentiometer. | ☐ P ☐ F |

# Part C — Functional test (hands-on)

Do TC-C-01 first Every test below assumes the BCM is awake. If you skip TC-C-01 the lamps stay off by design and everything will look broken.

### TC-C-01 — Wake the BCM

| Requirement | SRS-PWR-001, SRS-PWR-003 |
| --- | --- |
| Precondition | TC-B-03 passed. Board powered. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Before touching anything, note which LEDs are lit. | Only the Door Lock lamp (PA9) is lit. All others off - this is correct: a sleeping vehicle keeps its lamps off. | ☐ P ☐ F |
| 2 | TAP the ignition button (PA0) - press and release within 0.8 s. | A short tap, not a long hold. | ☐ P ☐ F |
| 3 | Observe the Ignition lamp (PB0). | The Ignition lamp comes ON. The BCM is now in Run. | ☐ P ☐ F |
| 4 | TAP ignition again. | The Ignition lamp goes OFF - back to Sleep. | ☐ P ☐ F |
| 5 | TAP once more to wake it for the remaining tests. | Ignition lamp ON again. | ☐ P ☐ F |

### TC-C-02 — Lighting mode cycle

| Requirement | SRS-LIGHT-001 |
| --- | --- |
| Precondition | TC-C-01 passed, BCM awake. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | HOLD the ignition button for at least 0.8 s, then release. | DRL lamp (PB1) lights - Parking. | ☐ P ☐ F |
| 2 | Repeat the hold. | DRL stays lit - DRL mode. | ☐ P ☐ F |
| 3 | Repeat the hold. | Low beam (PB10) lights as well. | ☐ P ☐ F |
| 4 | Repeat the hold. | High beam (PB11) lights too, WITH low beam still on. High beam must never run alone. | ☐ P ☐ F |
| 5 | Repeat the hold. | All lighting lamps go out - back to Off. | ☐ P ☐ F |

### TC-C-03 — Indicators

| Requirement | SRS-IND-001, SRS-IND-004 |
| --- | --- |
| Precondition | BCM awake. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Tap the left indicator button (PA1). | Left indicator lamp (PB12) FLASHES, starting lit. The right lamp stays off. | ☐ P ☐ F |
| 2 | Count the flashes over 10 seconds. | About 15 flashes - roughly 1.5 Hz. | ☐ P ☐ F |
| 3 | Tap the left button again. | Flashing stops. | ☐ P ☐ F |
| 4 | Tap the right button (PA2). | Right lamp (PB13) flashes. | ☐ P ☐ F |
| 5 | Tap the left button while the right is flashing. | The right lamp stops and the left starts - direct changeover. | ☐ P ☐ F |

### TC-C-04 — Horn and door locking

| Requirement | SRS-DOOR-001, SRS-HORN-002 |
| --- | --- |
| Precondition | BCM awake. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Note the Door Lock lamp (PA9) state. | It is lit - doors start locked. | ☐ P ☐ F |
| 2 | Tap the door lock button (PA5). | The lamp goes out - unlocked. The DRL lamp lights briefly (welcome lighting) then goes out. | ☐ P ☐ F |
| 3 | Listen to the buzzer. | Two short chirps on unlock; one chirp on lock. | ☐ P ☐ F |
| 4 | Tap the door lock button again. | The Door Lock lamp comes back on - locked. | ☐ P ☐ F |

### TC-C-05 — Battery reading

| Requirement | SRS-SENS-001 |
| --- | --- |
| Precondition | BCM awake. tools\bcm.ps1 talk available. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Turn the potentiometer fully anti-clockwise. | — | ☐ P ☐ F |
| 2 | Run: tools\bcm.ps1 talk | GET_BATTERY reports a low percentage, near 0 %. | ☐ P ☐ F |
| 3 | Turn the potentiometer fully clockwise. | — | ☐ P ☐ F |
| 4 | Run tools\bcm.ps1 talk again. | GET_BATTERY reports a high percentage, near 100 %. The reading tracks the knob. | ☐ P ☐ F |

# Part D — Safety tests

These are the behaviours that make this a Body Control Module rather than a lamp driver. Each one must pass.

### TC-D-01 — Brake lamp has the highest priority

| Requirement | SRS-BRK-001, SRS-SAFETY-003 |
| --- | --- |
| Precondition | BCM awake. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Press and hold the brake button (PA4). | Brake lamp (PB15) lights immediately. | ☐ P ☐ F |
| 2 | While still holding brake, tap hazard. | The hazards flash AND the brake lamp stays lit. Brake is never displaced by another function. | ☐ P ☐ F |
| 3 | Release the brake button. | The brake lamp goes out - it follows the pedal exactly. | ☐ P ☐ F |
| 4 | Put the BCM to sleep (tap ignition), then press brake. | The brake lamp STILL lights, even asleep. Brake overrides everything, including the sleep blanking. | ☐ P ☐ F |

### TC-D-02 — Hazard overrides the indicators

| Requirement | SRS-IND-003 |
| --- | --- |
| Precondition | BCM awake, indicators idle. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Tap the left indicator button. | Left lamp flashes. | ☐ P ☐ F |
| 2 | Tap the hazard button (PA3). | BOTH indicator lamps and the hazard lamp (PB14) now flash - hazard has taken over. | ☐ P ☐ F |
| 3 | Tap the left indicator button. | Nothing changes. Hazard cannot be pre-empted. | ☐ P ☐ F |
| 4 | Tap the right indicator button. | Nothing changes. | ☐ P ☐ F |
| 5 | Tap hazard again. | Hazard stops; the indicators are usable again. | ☐ P ☐ F |

### TC-D-03 — Load shedding protects the safety lamps

| Requirement | SRS-PWR-004, SRS-SAFETY-003 |
| --- | --- |
| Precondition | BCM awake. Potentiometer turned up. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Hold ignition long-presses until Low beam and DRL are lit. | DRL and Low beam are lit. | ☐ P ☐ F |
| 2 | Tap the left indicator so it is flashing. | Left indicator flashing. | ☐ P ☐ F |
| 3 | Press and hold the brake button. | Brake lamp lit. | ☐ P ☐ F |
| 4 | SLOWLY turn the potentiometer down while watching the lamps. | DRL and High beam GO OUT, but the brake lamp and the indicator KEEP working. This is the key result: a failing battery never removes the lamps other road users depend on. | ☐ P ☐ F |
| 5 | Turn the potentiometer back up. | The shed lamps return. | ☐ P ☐ F |

### TC-D-04 — Sleep blanks the lamps but keeps the lock indicator

| Requirement | SRS-PWR-003 |
| --- | --- |
| Precondition | BCM awake with several lamps lit. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Turn on Low beam via ignition long-presses. | Low beam and DRL lit. | ☐ P ☐ F |
| 2 | Tap ignition briefly to shut down. | The BCM shuts down. | ☐ P ☐ F |
| 3 | Wait 1 second and inspect every LED. | Every lamp is off EXCEPT the Door Lock lamp, which stays lit so a parked vehicle still shows it is secured. | ☐ P ☐ F |

### TC-D-05 — Reverse lamp is gated on gear

| Requirement | SRS-REV-001, SRS-SAFETY-005 |
| --- | --- |
| Precondition | BCM awake. Diagnostic tool connected (Part E) or talk available. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | In the diagnostic tool, click the 'Reverse' lamp button. | The command is accepted with status OK. | ☐ P ☐ F |
| 2 | Observe the physical Reverse LED (PA8). | The LED stays DARK. No reverse-gear switch is wired, so the arbiter refuses it. A diagnostic host can exercise a lamp but cannot override a safety rule. This is a PASS. | ☐ P ☐ F |

# Part E — Diagnostic tool (Windows GUI)

### TC-E-01 — Launch and connect

| Requirement | BCM-ICD-001 |
| --- | --- |
| Precondition | TC-B-04 passed. 'tools\bcm.ps1 talk' is NOT running. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Run: tools\bcm.ps1 desktop | A window titled 'BCM Diagnostic Tool' opens. | ☐ P ☐ F |
| 2 | Select the COM port in the drop-down. | Your adapter's port, e.g. COM13. | ☐ P ☐ F |
| 3 | Click Connect. | The status dot turns GREEN. | ☐ P ☐ F |
| 4 | Read the Firmware field. | It shows v1.0.0. | ☐ P ☐ F |

### TC-E-02 — Live monitoring

| Requirement | SRS-COM-003, SRS-DIAG-003 |
| --- | --- |
| Precondition | TC-E-01 passed. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Press and hold each button on the board in turn. | The matching indicator in the Inputs panel lights while held. | ☐ P ☐ F |
| 2 | Turn the potentiometer. | The Battery bar and percentage follow the knob. | ☐ P ☐ F |
| 3 | Tap ignition and watch the Power state field. | It changes Sleep → Wake → Run. | ☐ P ☐ F |
| 4 | Press several buttons and watch the Event log. | Entries appear with timestamps; faults appear in red. | ☐ P ☐ F |

### TC-E-03 — Driving outputs and reading faults

| Requirement | SRS-COM-003, SRS-DIAG-002 |
| --- | --- |
| Precondition | TC-E-01 passed, BCM awake. |

| # | Action | Expected result | Pass? |
| --- | --- | --- | --- |
| 1 | Click the 'DRL' lamp button in the Lamps panel. | The physical DRL LED lights. | ☐ P ☐ F |
| 2 | Click it again. | It goes out. | ☐ P ☐ F |
| 3 | Click 'Lock doors'. | The doors lock, the buzzer chirps, the Door Lock LED lights. | ☐ P ☐ F |
| 4 | Click 'Clear DTCs'. | The fault list empties. | ☐ P ☐ F |
| 5 | Turn the potentiometer fully down and wait 2 seconds. | '0x01 Battery low' appears in the fault list. | ☐ P ☐ F |

# Part F — Troubleshooting

| Symptom | Most likely cause | Action |
| --- | --- | --- |
| No LEDs respond to any button, but the GUI shows the button presses | The BCM is asleep. This is by design, not a fault. | Run TC-C-01. Tap ignition BRIEFLY (under 0.8 s). |
| Tapping ignition does nothing | You are holding it too long, so it changes the lighting mode instead of toggling power. | Press and release quickly, well under 0.8 s. |
| 'unable to connect to the target' | Board unpowered, SWDIO/SWCLK swapped, or no shared ground. | Check the board's power LED FIRST. Then reseat the four SWD wires. |
| No LEDs light at all, ever | Resistors too large, LEDs reversed, or the ground rail is not connected to a Blue Pill GND pin. | Use 220 Ω–1 kΩ. Long LED leg toward the GPIO. Check the ground jumper. |
| One LED dark, the rest fine | That LED is inserted backwards or its wire is open. | Reverse the LED; reseat the jumper. |
| A button does nothing | 3.3 V is not reaching the GPIO when pressed, or its 10 kΩ does not land on ground. | Check the GUI Inputs panel - if it does not light there either, it is wiring. |
| 'no serial port found' | The USB-UART adapter is not connected. The ST-Link does NOT carry this link. | Plug in the separate USB-UART adapter. |
| 'talk' times out | RX/TX not crossed, or wired to PA9/PA10 instead of PB6/PB7. | Adapter RX to PB6, adapter TX to PB7. |
| Diagnostic tool will not connect | Another program holds the COM port. | Close any window running 'talk'; only one may use the port. |
| Tool exits with 'You must install .NET' | The SDK is installed per-user, so the app cannot find its runtime. | Launch it with tools\bcm.ps1 desktop, which sets DOTNET_ROOT. |
| Firmware hangs under QEMU | Wrong QEMU. Version 9.x does not emulate the STM32F1 clock. | Use the gnuarmeclipse 2.8 build. |
| The board's micro-USB port does nothing | Normal. The STM32F103 has no USB bootloader in ROM. | That port supplies power only. Flash via ST-Link. |

# Part G — Results summary

Record the outcome of every test case.

| Test | Description | Result | Notes |
| --- | --- | --- | --- |
| TC-A-01 | Unit tests (189 cases) | ☐ Pass ☐ Fail |  |
| TC-A-02 | SIL scenarios (15) | ☐ Pass ☐ Fail |  |
| TC-A-03 | Protocol conformance (17) | ☐ Pass ☐ Fail |  |
| TC-A-04 | Emulated run | ☐ Pass ☐ Fail |  |
| TC-B-01 | Power and connections | ☐ Pass ☐ Fail |  |
| TC-B-02 | Programme the firmware | ☐ Pass ☐ Fail |  |
| TC-B-03 | CPU executing | ☐ Pass ☐ Fail |  |
| TC-B-04 | Protocol over the real link | ☐ Pass ☐ Fail |  |
| TC-C-01 | Wake the BCM | ☐ Pass ☐ Fail |  |
| TC-C-02 | Lighting mode cycle | ☐ Pass ☐ Fail |  |
| TC-C-03 | Indicators | ☐ Pass ☐ Fail |  |
| TC-C-04 | Horn and door locking | ☐ Pass ☐ Fail |  |
| TC-C-05 | Battery reading | ☐ Pass ☐ Fail |  |
| TC-D-01 | Brake lamp priority | ☐ Pass ☐ Fail |  |
| TC-D-02 | Hazard overrides indicators | ☐ Pass ☐ Fail |  |
| TC-D-03 | Load shedding protects safety lamps | ☐ Pass ☐ Fail |  |
| TC-D-04 | Sleep blanking | ☐ Pass ☐ Fail |  |
| TC-D-05 | Reverse gated on gear | ☐ Pass ☐ Fail |  |
| TC-E-01 | Tool launch and connect | ☐ Pass ☐ Fail |  |
| TC-E-02 | Live monitoring | ☐ Pass ☐ Fail |  |
| TC-E-03 | Driving outputs, reading faults | ☐ Pass ☐ Fail |  |

## Known limitations — not defects

The following are documented limitations of version 1.0.0. Do not raise them as failures.

Reverse-gear, door-ajar and light-mode switches, and the parking lamp, are not wired. Those inputs are inactive on hardware; they are covered by the SIL scenarios instead.

No LDR is fitted, so auto-headlight reports full daylight on the board. It is verified in the SIL.

Configuration is not stored in flash, so tunables reset on power cycle.

The lighting mode is changed by holding the ignition button, because no dedicated light switch is wired.

QEMU timing is approximate and must not be used to judge blink rates.

| Overall result | ☐ PASS ☐ FAIL |
| --- | --- |
| Tester signature |  |
| Date |  |

Reference documents: BCM-SRS-001 (requirements), BCM-ICD-001 (protocol), BCM-TST-001 (test report), BCM-PRJ-001 (project report), BCM-MAN-001 (user manual).